LAB 4

Victor Mwai SCT 212-0202/2021

April 7, 2025

# Solutions

E 1: Individual

Problem

Assume we have a computer where the CPI is 1.0 when all memory accesses (including data and instruction accesses) hit in the cache. The cache is a unified (data + instruction) cache of size 256 KB, 4-way set associative, with a block size of 64 bytes. The data accesses (loads and stores) constitute 50% of the instructions. The unified cache has a miss penalty of 25 clock cycles and a miss rate of 2%. Assume 32-bit instruction and data addresses.

a. What is the tag size for the cache?

b. How much faster would the computer be if all memory accesses were cache hits?

Answer

The number of bits needed for the block offset is calculated as log (64) = 6 bits.

The total number of sets in the cache is 256 KB ÷ (64 bytes × 4 ways) = 1024 sets, or 1K.

So, the number of index bits is log (1024) = 10 bits.

This means the number of tag bits is 32 - (10 + 6) = 16 bits, assuming 32-bit addresses.

Now for performance:

- The overall CPI (Cycles Per Instruction) is the sum of the base execution CPI and the extra stall cycles due to cache misses:

CPI = CPI_execution + StallCycles_per_Instruction

- If the cache always hits, then CPI = 1.

- But when there’s a miss rate, we calculate the stalls like this:

StallCycles per Instruction = (average memory accesses per instruction) × miss rate × miss penalty

On average, each instruction has:

  - 1 access for the instruction itself

  - 0.5 access for data (assuming 50% of instructions are loads/stores)

So: 1 + 0.5 = 1.5 accesses per instruction

Then: StallCycles = 1.5 × 0.02 × 25 = 0.75

Therefore: CPI = 1 + 0.75 = 1.75

E 2: Groups 2

Problem

You purchased an Acme computer with the following features:

• 95% of all memory accesses are found in the cache.

• Each cache block is two words, and the whole block is read on any miss.

• The processor sends references to its cache at the rate of 109 words per second.

• 25% of those references are writes.

• Assume that the memory system can support 109 words per second, reads or writes.

• The bus reads or writes a single word at a time (the memory system cannot read or

write two words at once).

• Assume at any one time, 30% of the blocks in the cache have been modified.

• The cache uses write allocate on a write miss.

You are considering adding a peripheral to the system, and you want to know how much

of the memory system bandwidth is already used. Calculate the percentage of memory system bandwidth used on the average in the two cases below. Be sure to state your assumptions.

a. The cache is write through.

b. The cache is write back.

Answer

The cache miss rate is 5% (or 0.05).

Each cache block holds 2 words, which equals 8 bytes.

The processor sends 10⁹ memory operations per second.

25% of these memory operations are writes, so there are 0.25 × 10⁹ writes/sec.

The bus is only capable of transferring one word at a time to or from memory.

30% of the cache blocks are modified and would need to be written back when evicted (for write-back caches).

The cache follows a write-allocate policy, meaning that on a write miss, the block is loaded into the cache before the write.

Breakdown of Access Fractions:

- Read hits = 75% (reads) × 95% (hit rate) = 0.7125

- Read misses = 75% × 5% = 0.0375

- Write hits = 25% × 95% = 0.2375

- Write misses = 25% × 5% = 0.0125

a. Write-Through Cache Behaviour:

- Read hit: No access to main memory.

- Read miss: Memory sends 2 words (entire block) to cache.

- Write hit: One word is written directly to memory.

- Write miss: Memory first sends 2 words to cache, then cache writes 1 word to memory.

Average words transferred = 0.7125 ∗ 0 + 0.0375 ∗ 2 + 0.2375 ∗ 1 + 0.0125 ∗ 3 = 0.35

Average bandwidth used = 0.35 ∗ 109

- Bandwidth used = 0.35 × 10⁹ words/sec

- So, the fraction of memory bandwidth used = 0.35 × 10⁹ / 10⁹ = 0.35 (or 35%)

b. Write-Back Cache Behaviour:

- Read hit and write hit: No memory access.

- Read miss:

  - If the block being replaced is modified, write 2 words back to memory, then load 2 words from memory.

  - If the block is clean, just load 2 words from memory.

- Write miss: Same as read miss, since it's write-allocated.

Average words transferred = 0.7125 ∗ 0 + 0.0375 ∗ (0.7 ∗ 2 + 0.3 ∗ 4) + 0.2375 ∗ 0 + 0.0125 ∗ (0.7 ∗ 2 + 0.3 ∗ 4) = 0.13 Average bandwidth used = 0.13 ∗ 109

- Bandwidth used = 0.13 × 10⁹ words/sec

- So, the fraction of memory bandwidth used =
0.13 × 10⁹ / 10⁹ = 0.13 (or 13%)

Comparison: From the calculations above, we can see that the write-through cache consumes more than twice the memory bandwidth compared to the write-back cache. This makes write-back more efficient in terms of bandwidth usage.

E 3: Groups of 2

Problem

One difference between a write-through cache and a write-back cache can be in the time it takes to write. During the first cycle, we detect whether a hit will occur, and during the second (assuming a hit) we actually write the data. Let’s assume that 50% of the blocks are dirty for a write-back cache. For this question, assume that the write buffer for the write through will never stall the CPU (no penalty). Assume a cache read hit takes 1 clock cycle, the cache miss penalty is 50 clock cycles, and a block write from the cache to main memory takes 50 clock cycles. Finally, assume the instruction cache miss rate is 0.5% and the data cache miss rate is 1%. Assuming that on average 26% and 9% of instructions in the workload are loads and stores, respectively, estimate the performance of a write-through cache with a two-cycle write versus a write-back cache with a two-cycle write.

Answer

The general equation for CPU performance is:

CPU Time = Instruction Count (IC) × Cycles Per Instruction (CPI) × Clock Cycle Time

CPI can be split into:

CPI = Base CPI (execution) + Stalls per instruction due to memory delays

Given Parameters:

- Instruction cache miss penalty: 50 cycles

- Data read hit latency: 1 cycle

- Data write hit latency: 2 cycles

- For write-through caches: Data miss penalty = 50 cycles

- For write-back caches: Data miss penalty = 50 or 100 cycles

- Data cache miss rate (MRD) = 1%

- Instruction cache miss rate (MRI) = 0.5%

- In write-back caches, 50% of blocks are dirty (need to be written back on eviction)

- Instruction mix:

  - 26% are loads

  - 9% are stores

CPI_execution = (26% × 1 cycle) + (9% × 2 cycles) + (65% × 1 cycle)

= 0.26 + 0.18 + 0.65

= 1.09

Write-through cache

Stall cycles due to cache misses:

Stall per instruction = MRI × 50

+ MRD × (Load% × 50 + Store% × 50)

= 0.005 × 50 + 0.01 × (0.26 × 50 + 0.09 × 50)

= 0.25 + 0.175 = 0.425

Total CPI: CPI = 1.09 + 0.425 = 1.515

Write-Back Cache

Here, on a miss, half the time we incur a 50-cycle penalty, and the other half, 100 cycles (due to dirty block eviction):

Stall per instruction = MRI × 50

+ MRD × (Load% × Avg penalty + Store% × Avg penalty)

= 0.005 × 50 + 0.01 × (0.26 × 75 + 0.09 × 75)

= 0.25 + 0.2625 = 0.5125

Total CPI: CPI = 1.09 + 0.5125 = 1.6025

Conclusion:

Comparing the two results:

- Write-through cache CPI = 1.515

- Write-back cache CPI = 1.6025

This shows that using a write-back cache makes the CPU roughly 6% slower in this scenario.